ADMLC Wildfire Modelling Seminar – Met Office, Exeter, 13th February 2025

Executive Summary, 3/2/2026

Authors: Matthew Wright, Ed Huckle, Andrew Kibble, Simon Gant, Peter Bedwell, Frances Beckett, Ben Drummond

Introduction

The Atmospheric Dispersion Modelling Liaison Committee (ADMLC) consists of representatives of Government Departments and Agencies, and other organisations with an interest in dispersion modelling of routine releases, and scenarios resulting from accidents and incidents. This includes releases of Chemical, Biological, Radiological and Nuclear (CBRN) material, air quality issues and other dispersion-related topics of significant national interest.

A key aim of the ADMLC is to review current modelling capabilities and limitations in a given area, and where possible to share findings with the dispersion modelling community and to inform regulatory agencies, dutyholders, consultants, academia and organisations involved in emergency planning and response. In this context, the ADMLC (alongside partners at the Met Office and UK Health Security Agency) organised a seminar to discuss the state-of-the-art in wildfire modelling on 13 February 2025. The motivation was to bring together national and international wildfire modelling experts to discuss novel advances and fundamental techniques, and identify knowledge gaps in modelling the dispersion of pollutants from wildfires.

The frequency and severity of wildfires across the globe, including in the UK, is likely to increase in the future due to climate change. Rapid forecasts are essential in both fire prediction and spread, and in the modelling of dispersion of smoke, ash and embers, in order for agencies to respond quickly and effectively to the threat to health, communities, infrastructure and landscapes (and associated economic impacts) both directly from fires and indirectly from exposure to wildfire smoke pollutants. Key challenges include the collection/generation and use of ground based observations (including photographs, air and satellite-based measurements) and numerical weather prediction (NWP) data; and the development, validation and application of new models or improved definition of key parameters in existing models e.g. plume rise, atmospheric chemistry and fuel/emissions characteristics in the UK setting.

Background

There is significant current government, public and media interest in wildfires at both a national and international level. A recent high profile example was the 2024-25 Los Angeles wildfire, which by the end of January 2025 had killed 29 people, damaged more than 20,000 properties and caused the evacuation of more than 200,000 people from their homes. The UK is also experiencing increased impact of wildfires and faces specific problems relating to the lack of current understanding of wildfire parameters in the UK context. In 2025 the UK saw the most fires and highest burnt area since at least 2006 [EFFIS], noting that these data only include fires > 30 ha, therefore likely underestimate burnt area and particularly number of fires. This is further compounded by the proximity of large population centres near to wildfire ‘hotspots’ (e.g. South Wales, South Pennines) leading to a large number of incidents at the urban-rural interface. It is recognised that wildfire smoke reduces air quality, increasing morbidity and mortality. Additional burdens include direct injuries, mental health impacts, co-exposures with other factors including heat, drought, ozone, water quality, soil quality and stability, biodiversity and habitat loss, increased future risk of flooding or landslides, and potentially also additional risks elsewhere from diversion of limited emergency response resources.

The role of dispersion modelling within the overall response to wildfires is a vital component to inform decision making and emergency planning and response, as well as to contribute to estimating potential public exposures and health risk. For retrospective assessments, modelling is required as measurements can never be entirely comprehensive spatially and temporally, whilst for prospective assessments modelling is the primary basis. Accurate modelling can provide valuable data to aid public health interventions such as advice/decisions to shelter or evacuate and to support public health messaging; performing controlled/prescribed burns to reduce further risk to the public; and to help identify populations at immediate or potential risk, for specific events or more generally. In the UK context, particular attention needs to be paid to assessing smoke composition and fire/plume behaviour for typical fuels and weather conditions, and to the particular topographies of common UK wildfire locations. Better-studied regions worldwide have very different topographies and fuel makeup and in many cases are on a vastly larger scale. It is important to understand how research and practice in the wider global context can be translated to UK wildfire modelling and response.

Poor air quality driven by wildfire smoke has the potential to impact many more people than direct danger from the fire itself. In the Los Angeles wildfire, one monitoring network station recorded a PM2.5 concentration of 483.7 µg m-3, over ten times the (current) daily average US limit value of 35 µg m-3. In 2018, the UK experienced one of its largest recorded wildfires to date on Saddleworth Moor. Air monitoring data showed a 7-fold increase in PM2.5 concentration at sites ~ 8 miles from the fire itself, compared to background levels. Wildfires emit a highly complex mixture of particulates and gases, which can react to form a very wide range of secondary components with potentially different toxicities. The composition varies with factors such as source material, meteorological conditions, burn type/temperature etc. (which is itself dependent on suppression activities, if any) and hence is both difficult to characterise and often highly variable between events, and even over the duration of a single event. As a result, the composition of wildfire smoke in general, and that to be expected from any given wildfire event, is not well understood.

Uncertainties involved in estimating exposure concentrations and doses via dispersion modelling and/or monitoring leads to challenges in performing toxicological and quantitative public health risk assessments and providing forecasts and related public health advice. This was highlighted in the Health Effects of Climate Change (HECC) report, which reviewed evidence relating to wildfire smoke and identified several research and knowledge gaps. Priority areas identified include further study of smoke composition, emission rates and toxicology, and improved exposure assessment via both monitoring and modelling.

Seminar overview

The seminar presentations covered a range of studies and models at an advanced technical level, and outlined world-leading research and operational wildfire modelling and forecast programmes. Specific topics included:

- Modelling of emissions, dispersion and health effects associated with agricultural biomass burning in the US,

- Development of a novel model for fire behaviour and propagation, which can be coupled with atmospheric dispersion models and explicitly accounts for the influence of fuel type on pyrocumulus formation,

- A retrospective reanalysis of the Saddleworth Moor fires of 2018 to assess the public health impact of the downwind smoke plume, and

- The implementation of wildfire emissions and smoke transport models within broader atmospheric monitoring networks in Europe (Copernicus Atmospheric Monitoring Service), Canada (Meteorological Service of Canada) and the US (NOAA Air Resources Laboratory).

Technical details were given around the dispersion models used e.g. the class of model, appropriate distance and time scales and grid size; the inputs to these models e.g. meteorological observations and/or NWP models, burn-related parameters including fuel type/land use, fuel consumption rate or fire radiative power, processes included within the model e.g. plume rise and injection height, pyrocumulus formation, and outputs including airborne concentrations of a range of pollutants and wet and dry deposition.

Discussion of key issues

Of particular note was the need for better observations during wildfire events, which might include photography, satellite imagery and drone footage, alongside better (static or mobile) air quality monitoring, to better constrain important parameters within models (e.g. plume rise, burn area) both spatially and temporally. This would also provide vital information to validate existing or new models, especially where these have been developed in a non-UK context but find application here. For example, satellite-derived data are commonly used in other countries to feed into air quality models which incorporate wildfires as a source. However, many of the small-scale wildfires in the UK are not detected by satellites – Forestry Commission data show that 99% of fires in England are < 1 ha. Even smaller fires have potentially important implications for public health, given the close proximity of fires to urban areas in the UK. This points to the need for alternative, innovative ways of monitoring UK wildfires.

Related to this was the call for improved measurement of pollutant concentrations downstream of wildfires. Discussion focused specifically on PM2.5 as a regulated quantity in terms of air quality, which has reasonably well-understood negative health impacts. A significant step forward would be the development of UK-specific emissions factors for specific chemical components within wildfire smoke, and assessment of their toxicological impact.

Identifying which models, data sets etc. already in use or under development globally are applicable in the UK setting is also important to quickly build the knowledge base in this area. For example, the significant body of existing work on forest fires (including prescribed burns and in laboratory tests with a range of fuels from different species) to determine emission factors, may yield information relevant to typical UK woodland wildfires. However, these results may not represent other types of fires experienced in the UK e.g. peat fires, which are less commonly studied in a global context.

The importance of ensuring that research and model development has applicability to operational use in the UK context was also raised, with stakeholder and end-user needs, and incorporation of means of evaluating progress, considered throughout this process. This includes, for example, tailoring model development to both rapid response and more detailed scientific contexts, where the former would be deployed quickly during an incident to inform response and public health advice, while the latter might be used to obtain accurate prediction of impacts post-hoc, to help formulate emergency planning or preventative actions, or to evaluate potential high-risk scenarios.

An evaluation of what is needed in each of these areas, in both the immediate and longer term, will drive future research and help to develop improved dispersion models for wildfires. In turn this will allow the wildfire community to highlight the importance of the topic within government, academia and among public and environmental health responders/practitioners, and to build UK capacity for predicting, understanding, responding to and reducing the impact of wildfires in the future.

Link to seminar slides and recordings on ADMLC website

https://admlc.com/events/